# Consolidated Test Report

Test Types: Network Throughput (iperf), VIOS Resilience

VIOS Tested: VIOS71B
Client Hosts: aix14113

Report Generated: 2026-09-17 16:47:10

# Table of Contents

1. Network Throughput Test Results

1.1 pha12

2. VIOS Resilience Test

2.1 Network Resilience Test

2.1.1 Before Shutdown VIOS VIOS71B (aix14113)

2.1.2 During Shutdown VIOS VIOS71B (aix14113)

2.1.3 After Shutdown VIOS VIOS71B (aix14113)

2.2 Disk/SAN Path Resilience Test

2.2.1 Before Shutdown VIOS VIOS71B (aix14113)

2.2.2 During Shutdown VIOS VIOS71B (aix14113)

2.2.3 After Shutdown VIOS VIOS71B (aix14113)

3. Summary

4. Appendix & Logs

# 1. Network Throughput Test Results

iperf TCP bandwidth measurements between AIX LPARs.

| # | Client | Server IP | Date | TCP Bandwidth |
| --- | --- | --- | --- | --- |
| 1 | pha12 | 192.168.141.13 | Thu Sep 17 16:16:49 HKT 2026 | 12.5 Gbits/sec |

## 1.1 pha12

| Client Hostname | pha12 |
| --- | --- |
| AIX Version | 7200-05-11-2546 |
| Target Server IP | 192.168.141.13 |
| iperf Port | 5201 |
| Test Duration | 10s per test |
| Test Date | Thu Sep 17 16:16:49 HKT 2026 |

### TCP Throughput

| TCP Window Size | 256 KByte (default) |
| --- | --- |
| Transfer | 14.6 GBytes |
| Bandwidth | 12.5 Gbits/sec |

TCP Throughput Test
================================================================================

[INFO] Testing TCP throughput...
------------------------------------------------------------
Client connecting to 192.168.141.13, TCP port 5201
TCP window size:  256 KByte (default)
------------------------------------------------------------
[  3] local 192.168.141.12 port 32794 connected with 192.168.141.13 port 5201
[ ID] Interval       Transfer     Bandwidth
[  3]  0.0-10.0 sec  14.6 GBytes  12.5 Gbits/sec


================================================================================

## 1.2 Server-Side Logs

### iperf_srv_iperf_server.log

Server bandwidth: 12.5 Gbits/sec  (Transfer: 14.6 GBytes)

------------------------------------------------------------
Server listening on TCP port 5201
TCP window size:  640 KByte (default)
------------------------------------------------------------
[  4] local 192.168.141.13 port 5201 connected with 192.168.141.12 port 32794
[ ID] Interval       Transfer     Bandwidth
[  4]  0.0-10.0 sec  14.6 GBytes  12.5 Gbits/sec

# 2. VIOS Resilience Test

This section validates that AIX LPARs maintain continuous service availability through redundant VIOS paths during a VIOS shutdown and restart cycle.

VIOS under test: VIOS71B

Client hosts: aix14113

## 2.1 Network Resilience Test

Ping monitoring results and network adapter/interface snapshots captured before, during, and after VIOS shutdown.

### 2.1.1 Before Shutdown VIOS VIOS71B  (Host: aix14113)

VIOS went DOWN at: 16:18:50  │  VIOS came UP at: 16:27:24  │  Total downtime: 8m 34s

No data recorded in this phase.

Network Snapshot:

► entstat en0 (link/state)

ETHERNET STATISTICS (en0) :
Device Type: Virtual I/O Ethernet Adapter (l-lan)
Hardware Address: 26:fc:4f:1c:cf:02
Transmit Errors: 0                            Receive Errors: 0
Packets Dropped: 0                            Packets Dropped: 79382
                                              Bad Packets: 0

► Ping test en0 -> 192.168.141.12

PING 192.168.141.12: (192.168.141.12): 56 data bytes
64 bytes from 192.168.141.12: icmp_seq=0 ttl=255 time=0 ms
64 bytes from 192.168.141.12: icmp_seq=1 ttl=255 time=0 ms
64 bytes from 192.168.141.12: icmp_seq=2 ttl=255 time=0 ms

--- 192.168.141.12 ping statistics ---
3 packets transmitted, 3 packets received, 0% packet loss
round-trip min/avg/max = 0/0/0 ms

### 2.1.2 During Shutdown VIOS VIOS71B  (Host: aix14113)

VIOS went DOWN at: 16:18:50  │  VIOS came UP at: 16:27:24  │  Total downtime: 8m 34s

| Metric | Value |
| --- | --- |
| Total Checks | 179 |
| OK | 179 (100%) |
| LOSS (Packet Loss) | 0 (0%) |

Time range: 16:18:56 - 16:21:55 (179 samples)

First 5 entries:

16:18:56 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=1 ms
16:18:57 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms
16:18:58 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms
16:18:59 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms
16:19:00 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms

Last 5 entries:

16:21:51 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms
16:21:52 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms
16:21:53 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms
16:21:54 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms
16:21:55 OK  64 bytes from 192.168.140.254: icmp_seq=0 ttl=64 time=0 ms

Network Snapshot:

► entstat en0 (link/state)

ETHERNET STATISTICS (en0) :
Device Type: Virtual I/O Ethernet Adapter (l-lan)
Hardware Address: 26:fc:4f:1c:cf:02
Transmit Errors: 0                            Receive Errors: 0
Packets Dropped: 0                            Packets Dropped: 79382
                                              Bad Packets: 0

► Ping test en0 -> 192.168.141.12

PING 192.168.141.12: (192.168.141.12): 56 data bytes

--- 192.168.141.12 ping statistics ---
3 packets transmitted, 0 packets received, 100% packet loss

### 2.1.3 After Shutdown VIOS VIOS71B  (Host: aix14113)

VIOS went DOWN at: 16:18:50  │  VIOS came UP at: 16:27:24  │  Total downtime: 8m 34s

No data recorded in this phase.

Network Snapshot:

► entstat en0 (link/state)

ETHERNET STATISTICS (en0) :
Device Type: Virtual I/O Ethernet Adapter (l-lan)
Hardware Address: 26:fc:4f:1c:cf:02
Transmit Errors: 0                            Receive Errors: 0
Packets Dropped: 0                            Packets Dropped: 79382
                                              Bad Packets: 0

► Ping test en0 -> 192.168.141.12

PING 192.168.141.12: (192.168.141.12): 56 data bytes
64 bytes from 192.168.141.12: icmp_seq=0 ttl=255 time=0 ms
64 bytes from 192.168.141.12: icmp_seq=1 ttl=255 time=0 ms
64 bytes from 192.168.141.12: icmp_seq=2 ttl=255 time=0 ms

--- 192.168.141.12 ping statistics ---
3 packets transmitted, 3 packets received, 0% packet loss
round-trip min/avg/max = 0/0/0 ms

## 2.2 Disk/SAN Path Resilience Test

Disk I/O monitoring results and MPIO/SAN path snapshots captured before, during, and after VIOS shutdown.

### 2.2.1 Before Shutdown VIOS VIOS71B  (Host: aix14113)

VIOS went DOWN at: 16:18:50  │  VIOS came UP at: 16:27:24  │  Total downtime: 8m 34s

No data recorded in this phase.

Disk/MPIO Snapshot:

► MPIO path status (all disks)

Enabled hdisk0 fscsi0
Enabled hdisk0 fscsi0
Enabled hdisk0 fscsi1
Enabled hdisk0 fscsi1

► Paths for hdisk0

name   path_id parent connection         status

hdisk0 0       fscsi0 500507605e80cc01,0 Enabled
hdisk0 1       fscsi0 500507605e80cc21,0 Enabled
hdisk0 2       fscsi1 500507605e80cc01,0 Enabled
hdisk0 3       fscsi1 500507605e80cc21,0 Enabled

► MPIO attrs for hdisk0 (hcheck_interval => auto-recovery)

hcheck_interval 60             Health Check Interval True+
hcheck_mode     nonactive      Health Check Mode     True+
reserve_policy  no_reserve     Reserve Policy        True+
algorithm       shortest_queue Algorithm             True+

► lsmpio detail (if available)

Device           Vendor Id  Product Id       Size       Volume Name              
hdisk0           IBM        FlashSystem-9840   60.00GiB  -                      
name    path_id  status   path_status  parent  connection

hdisk0  0        Enabled  Sel          fscsi0  500507605e80cc01,0
hdisk0  1        Enabled  Sel          fscsi0  500507605e80cc21,0
hdisk0  2        Enabled  Sel          fscsi1  500507605e80cc01,0
hdisk0  3        Enabled  Sel          fscsi1  500507605e80cc21,0

► Children of fcs0 (fscsi devices)

fscsi0 Available C3-T0-01 FC SCSI I/O Controller Protocol Device
fscsi0 Available C3-T0-01 FC SCSI I/O Controller Protocol Device

► Children of fcs1 (fscsi devices)

fscsi1 Available C4-T0-01 FC SCSI I/O Controller Protocol Device
fscsi1 Available C4-T0-01 FC SCSI I/O Controller Protocol Device

### 2.2.2 During Shutdown VIOS VIOS71B  (Host: aix14113)

VIOS went DOWN at: 16:18:50  │  VIOS came UP at: 16:27:24  │  Total downtime: 8m 34s

| Metric | Value |
| --- | --- |
| Total Checks | 172 |
| OK | 172 (100%) |
| SLOW (Failover Stall) | 0 (0%) |
| FAIL | 0 (0%) |

Time range: 16:18:56 - 16:21:55 (172 samples)

First 5 entries:

16:18:56 OK  dd read from hdisk0 (0s)
16:18:57 OK  dd read from hdisk0 (0s)
16:18:58 OK  dd read from hdisk0 (0s)
16:18:59 OK  dd read from hdisk0 (0s)
16:19:00 OK  dd read from hdisk0 (0s)

Last 5 entries:

16:21:51 OK  dd read from hdisk0 (0s)
16:21:52 OK  dd read from hdisk0 (0s)
16:21:53 OK  dd read from hdisk0 (0s)
16:21:54 OK  dd read from hdisk0 (0s)
16:21:55 OK  dd read from hdisk0 (0s)

Disk/MPIO Snapshot:

► MPIO path status (all disks)

Enabled hdisk0 fscsi0
Enabled hdisk0 fscsi0
Failed  hdisk0 fscsi1
Failed  hdisk0 fscsi1

► Paths for hdisk0

name   path_id parent connection         status

hdisk0 0       fscsi0 500507605e80cc01,0 Enabled
hdisk0 1       fscsi0 500507605e80cc21,0 Enabled
hdisk0 2       fscsi1 500507605e80cc01,0 Failed
hdisk0 3       fscsi1 500507605e80cc21,0 Failed

► MPIO attrs for hdisk0 (hcheck_interval => auto-recovery)

hcheck_interval 60             Health Check Interval True+
hcheck_mode     nonactive      Health Check Mode     True+
reserve_policy  no_reserve     Reserve Policy        True+
algorithm       shortest_queue Algorithm             True+

► lsmpio detail (if available)

Device           Vendor Id  Product Id       Size       Volume Name              
hdisk0           IBM        FlashSystem-9840   60.00GiB  -                      
name    path_id  status   path_status  parent  connection

hdisk0  0        Enabled  Sel          fscsi0  500507605e80cc01,0
hdisk0  1        Enabled  Sel          fscsi0  500507605e80cc21,0
hdisk0  2        Failed   Deg,Fai,PFa  fscsi1  500507605e80cc01,0
hdisk0  3        Failed   Deg,Fai,PFa  fscsi1  500507605e80cc21,0

► Children of fcs0 (fscsi devices)

fscsi0 Available C3-T0-01 FC SCSI I/O Controller Protocol Device
fscsi0 Available C3-T0-01 FC SCSI I/O Controller Protocol Device

► Children of fcs1 (fscsi devices)

fscsi1 Available C4-T0-01 FC SCSI I/O Controller Protocol Device
fscsi1 Available C4-T0-01 FC SCSI I/O Controller Protocol Device

### 2.2.3 After Shutdown VIOS VIOS71B  (Host: aix14113)

VIOS went DOWN at: 16:18:50  │  VIOS came UP at: 16:27:24  │  Total downtime: 8m 34s

No data recorded in this phase.

Disk/MPIO Snapshot:

► MPIO path status (all disks)

Enabled hdisk0 fscsi0
Enabled hdisk0 fscsi0
Enabled hdisk0 fscsi1
Enabled hdisk0 fscsi1

► Paths for hdisk0

name   path_id parent connection         status

hdisk0 0       fscsi0 500507605e80cc01,0 Enabled
hdisk0 1       fscsi0 500507605e80cc21,0 Enabled
hdisk0 2       fscsi1 500507605e80cc01,0 Enabled
hdisk0 3       fscsi1 500507605e80cc21,0 Enabled

► MPIO attrs for hdisk0 (hcheck_interval => auto-recovery)

hcheck_interval 60             Health Check Interval True+
hcheck_mode     nonactive      Health Check Mode     True+
reserve_policy  no_reserve     Reserve Policy        True+
algorithm       shortest_queue Algorithm             True+

► lsmpio detail (if available)

Device           Vendor Id  Product Id       Size       Volume Name              
hdisk0           IBM        FlashSystem-9840   60.00GiB  -                      
name    path_id  status   path_status  parent  connection

hdisk0  0        Enabled  Sel          fscsi0  500507605e80cc01,0
hdisk0  1        Enabled  Sel          fscsi0  500507605e80cc21,0
hdisk0  2        Enabled  Deg          fscsi1  500507605e80cc01,0
hdisk0  3        Enabled  Deg          fscsi1  500507605e80cc21,0

► Children of fcs0 (fscsi devices)

fscsi0 Available C3-T0-01 FC SCSI I/O Controller Protocol Device
fscsi0 Available C3-T0-01 FC SCSI I/O Controller Protocol Device

► Children of fcs1 (fscsi devices)

fscsi1 Available C4-T0-01 FC SCSI I/O Controller Protocol Device
fscsi1 Available C4-T0-01 FC SCSI I/O Controller Protocol Device

# 3. Summary

## 3.1 Network Throughput Summary

| Client | Server IP | TCP BW | Result |
| --- | --- | --- | --- |
| pha12 | 192.168.141.13 | 12.5 Gbits/sec | PASS |

## 3.2 VIOS Resilience Summary

| Host | VIOS | Downtime | Ping Total | Ping Loss | Disk Slow/Fail | Result |
| --- | --- | --- | --- | --- | --- | --- |
| aix14113 | VIOS71B | 8m 34s | 179 | 0 (0%) | 0s / 0f | PASS |

## 3.3 Overall Conclusion

Network throughput tests captured TCP bandwidth measurements for all tested LPARs. All VIOS resilience tests completed successfully. During the VIOS shutdown periods, all AIX LPARs maintained continuous network connectivity and disk I/O availability through redundant VIOS paths. All MPIO paths recovered to Enabled state after VIOS restart.

# 4. Appendix & Logs

This appendix catalogues the raw test log files generated during each test. The full files are available in the testresult/ directory alongside this report and in the corresponding .tar.gz archives.

## 4.1 Network Throughput Logs

| File Name | Host | Key Metrics |
| --- | --- | --- |
| iperf_20260917_161649_result.log | pha12 | TCP: 12.5 Gbits/sec |
| iperf_srv_iperf_server.log | Server | BW: 12.5 Gbits/sec |

## 4.2 VIOS Resilience Logs

### Host: aix14113  —  VIOS: VIOS71B  (vios_res_aix14113_20260917_161748)

| File Name | Size |
| --- | --- |
| adapter_slots_20260917_161748.txt | 7.4 KB |
| after_restart_recovery_20260917_161748.txt | 1.4 KB |
| after_shutdown_pathdetect_20260917_161748.txt | 1.6 KB |
| inventory_20260917_161748.txt | 1.6 KB |
| monitor_disk_20260917_161748.txt | 6.7 KB |
| monitor_multinet_20260917_161748.txt | 1.8 KB |
| monitor_ping_20260917_161748.txt | 12.8 KB |
| snapshot_VIOS71B_after_restart_20260917_162724.txt | 11.9 KB |
| snapshot_VIOS71B_after_shutdown_20260917_162156.txt | 11.7 KB |
| snapshot_VIOS71B_before_shutdown_20260917_161749.txt | 11.9 KB |
| vios_res_client.log | 2.2 KB |